True / False

1. Short pants cut off above the knee are appropriate for masons in extremely hot weather. a.; True. b.; False. ANSWER:; False.

2. Hard hats are required only for construction jobs that have been determined to have excessive hazards. a.; True. b.; False. ANSWER:; False.

3. Because the spray can obstruct vision, goggles are not recommended when using a wet saw. a.; True. b.; False. ANSWER:; False.

4. If you constantly have to raise your voice to shout or be hear, the noise level is too high, and you should use ear plugs. a.; True. b.; False. ANSWER:; True.

5. Employers must also, according to the law, conduct safety training for their employees regarding hazardous chemicals or products in the work area. a.; True. b.; False. ANSWER:; True.

Multiple Choice

6. Lightweight plastic hats, commonly called _____, afford little protection and do not conform to regulations. a.; hard hats. b.; bump hats. c.; plastic caps. d.; hard shells. ANSWER:; b.

7. A hard hat has two parts: the _____ and the _____ . a.; metal; foam. b.; visor; casing. c.; skin; padding. d.; shell; suspension materials. ANSWER:; d.

8. Hearing protection should be worn when the noise exposure level _____ . a.; exceeds 90 decibels. b.; reaches 75 decibels. c.; is less than 80 decibels. d.; approaches 85 decibels. ANSWER:; a.

9. Mortar pans should be placed back from the walls approximately ___ to allow proper working space. a.; 5'. b.; 3'. c.; 2'. d.; 6". ANSWER:; c.

10. Safety nets made of _____ are stronger, lighter in weight, and more resistance to wear and weather elements than the original safety nets. a.; manilla rope. b.; chain mail. c.; synthetic fibers. d.; natural fibers. ANSWER:; c.

11. Grounded electrical plugs can be easily identified because they have _____ . a.; three prongs. b.; orange cords. c.; UL-tested tags. d.; triangular heads. ANSWER:; a.

12. After prolonged use, the head of a chisel becomes flared out from striking with a steel hammer, thus the chisel becomes _____ or burred. a.; gourded. b.; mashed. c.; cupped. d.; mushroomed. ANSWER:; d.

13. Protective safety rod caps should be made of _____ . a.; rigid plastic. b.; thick rubber. c.; cast concrete. d.; flexible plastic. ANSWER:; a.

14. _____ scaffolding can consist of one or more platforms supported by rigid, strong, load-bearing members or supports, such as tubular steel sections, poles, frames, masts, and outriggers. a.; Suspending. b.; Swinging. c.; Lifting. d.; Supporting. ANSWER:; d.

15. _____ are injuries to the muscular or skeletal framework parts of the body. a.; MSs. b.; MSFs. c.; WMSDs. d.; WRMDs. ANSWER:; c.

16. Which of the following statements is a good safety practice for masons? a.; Never use gasoline-operated mortar mixers.. b.; Wear rubber gloves when operating a wet masonry saw.. c.; Avoid angled shields when using a saw.. d.; Use lightweight household extension cords with grinders.. ANSWER:; b.

17. Any injury serious enough to require medical attention should be reported to the supervisor or instructor _____ . a.; immediately. b.; within 24 hours of the incident. c.; in written form by then end of the work/school week. d.; only if transportation to a medical facility is necessary. ANSWER:; a.

18. _____ is defined as any clothing or other equipment or devices workers wear to protect themselves from job hazards. a.; MDSDs. b.; OSHA. c.; PPE. d.; HAZMATs. ANSWER:; c.

19. In the case of an emergency when no dust mask or respirator is available, _____ will suffice until you can get out of the area. a.; a wet handkerchief tied snugly around your face covering your mouth and nose. b.; using your gloved hand to cover your mouth and nose. c.; pulling your shirt up and over your mouth and nose and holding tightly with your hand. d.; placing and holding your mouth and nose tightly in the bend of your elbow. ANSWER:; a.

20. Chemical manufacturers and importers must develop or obtain a material safety data sheet for _____ . a.; only chemicals that can burn or asphyxiate users. b.; any substance that can cause a chemcial reaction. c.; everything from hazardous chemicals to electrical equipment. d.; each hazardous chemical they produce or distribute. ANSWER:; d.